Sukkur Institute of Business Administration University

Department of Computer Science

ESE-201: Digital Logic Design Lab

Lab # 03: X-OR & X-NOR Gates

Note: Submit this lab hand-out in the next lab with attached solved activities and exercises

Submission Profile

Name: 	   Majid Ali					Submission date (24/08/2026):

Marks obtained:					Receiving authority name and signature:

Comments:

________________________________________________________________________________

Instructor Signature

Lab Learning Objectives:

Upon successful completion of this experiment, the student will be able:

- To implement and verify X-OR gate operations using 74LS86 IC & Module KL-33001

- To implement and verify X-NOR gate operations using 74LS266 IC & Module KL-33001

Lab Hardware and Software Required:

- 74LS86 IC (X-OR gate )

- 74LS266 IC(X-NOR gate)

- Module KL-33001

- Breadboard

- Connecting Wires

Background Theory:

X-OR gate:

X=A⊕B

Exclusive OR gate, commonly abbreviated as X-OR is a special type of OR gate. Logic symbol for the XOR gate is shown in Fig 3.1 (a & b). The output of an X-OR is high only when the two inputs are at opposite logic levels. Alternatively, X-OR gate provides a high output if one input or the other is high, but not both. The X-OR gate has only two inputs. Unlike the other gates, it never has more than two inputs. The X-OR output expression is written as X=AB, where the symbol” ” represented X-OR operation.

X=A⊕B

A

A

B

B

Fig 3.1: (a) Distinctive symbol of X-OR gate 	(b) Standard X-OR gate symbol

74LS86 2-Input X-OR gate IC:

In order to implement the X-OR operation using IC, the TTL 74LS86 2-input X-OR gate IC can be used. This IC has 14 pin Dual Inline Package (DIP) configuration as shown in fig 3.2. The power supply connections are made to pin 7 and 14. Pin 1 is identified by a small indented circle next to it or by a notch cut out between pin 1 and 14.

Fig 3.2: 74LS86 2-input X-OR gate IC pin configuration

X-NOR gate

Exclusive NOR (abbreviated as X-NOR) is the complement of X-OR, like the X-OR gate, the X-NOR gate has only two inputs. The logic symbol for X-NOR gate is shown in Fig 3.3 (a & b).

X=A⊕B

X=A⊕B

The characteristics of X-NOR will be completely opposite of X-OR gate. As in X-OR gate, the same inputs caused low output but here in X-NOR the case is reversed, means on same inputs the output is high.

A

A

B

B

Fig 3.3: (a) Distinctive symbol of 2-input X-NOR gate(b) Standard 2-input X-NOR gate symbol

74LS266 2-input X-NOR gate IC:

In order to implement the NOR gate operation using IC, the TTL 74LS266 2-input X-NOR gate IC can be used. This IC has 14 pin Dual Inline Package (DIP) configuration as shown in Fig 3.4. The power supply connections are made to pin 7 and 14. Pin 1 is identified by a small indented circle next to it or by a notch cut out between pin 1 and 14.

Fig 3.4: 74LS266 2-input X-NOR gate IC pin configuration

Lab Examples:

- Implementation of X-OR gate also Implement this into Multisim/TinkerCad.

- In order to implement the X-OR  gate operations with the help of ICs, take the 74LS86

- Pin Assignments for the ICs have been shown in Fig 3.2

- Take 74LS86 IC and insert it in the breadboard present on the KL-33001 training kit

- Connect its pin 14 to +5V and pin 7 to ground

- Use first gate (any one can be taken) from IC by connecting input pin 1 & input pin 2 to SW0 & SW1 respectively, and output pin3to LED and observe the AND gate operation

- Fill the following observation table

| Input |  | Output |  |
| --- | --- | --- | --- |
| A | B | LED (on / off) | Level ( 1 / 0 ) |
| 0 | 0 | 0ff | 0 |
| 0 | 1 | on | 1 |
| 1 | 0 | on | 1 |
| 1 | 1 | off | 1 |

Table 3.1: Observation table of 2-input X-OR gate

Lab Activities:

- Implementation of X-NOR gate also Implement this into Multisim/TinkerCad.

- In order to implement the X-NOR operations with the help of IC, take the 74LS266 IC

- Pin Assignments for the ICs have been shown in Fig 3.4

- Take 74LS266 IC and insert it in the breadboard present on the KL-33001 training kit

- Connect its pin 14 to +5V and pin 7 to ground

- Use first gate (any one can be taken) from IC by connecting input pin 1 & input pin 2 to SW0 & SW1 respectively, and output pin3to LED and observe the AND gate operation

- Fill the following observation table

| Input |  | Output |  |
| --- | --- | --- | --- |
| A | B | LED (on / off) | Level ( 1 / 0 ) |
| 0 | 0 | on | 1 |
| 0 | 1 | off | 0 |
| 1 | 0 | 0ff | 0 |
| 1 | 1 | on | 1 |

Table 3.2: Observation Table of 2-input X-NOR gate

Lab Exercises:

- XOR and XNOR Gate Applications also implement this into Multisim/Tinker Cad.

XOR

XNOR